Lab Section: Tues 3-5pm

Name: Elissa Ito

Collaborators (if any):

Lab 7 – Frequency Domain Control Design

Prelab

Experiment #1: Open and Closed Loop Bode Plots:

- Attach open loop Bode plots and locate the plant poles. Explain the discrepancies between the model and experimentally collected Bode plots.

0.172/0.15s^2+s

Poles at s=0, -6.6667 (where phase @-90, @-135)

Discrepancy at high and low frequencies – low frequency is deadband and high frequency is oversaturated.

- Attach closed loop Bode plots with a proportional controller with Kp=100. Explain the discrepancies between the model and measured bode plots.

Discrepancy at high and low frequencies – low frequency is deadband and high frequency is oversaturated.

- When the experiment and model do not agree, is this due to model, or the way we collected data? Explain your reasoning.

The way we collected data; the friction in the flywheel causes deadband and saturation and low and high frequencies respectively.

Experiment #2. Controller Design:

Time domain step response specifications: overshoot 7% or less, and 2% settling time of 0.4 seconds or less.

%OS<=7 => zeta >= 0.646

PM  100*zeta = 64.6

0.4 = 4/(zeta*w_n) => w_n = 15.48

w_c  w_n = 15.48

phi_max = 64.6-27.5 = 37.1

- What values of ɸm and ωc should we use to achieve the required time domain performances?

PM= 64.6

ωc= 15.48

- Transfer function of your lead controller:

Gcs=487.51(s+7.218)/(s+33.2)=106.01(0.14s+1)/(0.03s+1)

- Attach the Bode plots of the simulated responses with the new controller.

- Find the closed-loop step response characteristics by running the MATLAB command:

stepinfo(feedback(Gc*Gp,1))

RiseTime: 0.0872

SettlingTime: 0.2777

SettlingMin: 0.9138

SettlingMax: 1.0651

Overshoot: 6.5078

Undershoot: 0

Peak: 1.0651

PeakTime: 0.1850

- Include the SISOTOOL screenshot showing the root locus of the compensated system.

Experiment #3. Controller Testing:

- Attach open loop Bode plots for the compensated system, (GcGp) and determine the new crossover frequency and phase margin from the actual data.

Crossover Freq = 15.4

Phase Margin = 64

- Attach the sine wave plot when driven by a frequency equals to the crossover frequency, and include the phase and magnitude information for the crossover frequency. Do they match with the values determined from the open loop Bode plots?

Gain = (2.1983+0.3734)/(1.0461+1.0452) = 1.23 = 1.798dB

Phase = 360*(0.1002-0.2306)*2.451=-115.06deg

- Attach the closed loop Bode plots and estimate the closed loop bandwidth.

Bandwidth = 33.5

- Attach the step response of the actual closed loop system. Measure the actual percent overshoot and settling time.

%OS = 3.7

Ts = 0.22s

- Describe the effects of incorporating the lead controller onto the system.

Incorporating the lead controller caused the step response to reach the transient response a lot faster. It caused a greater overshoot.